Cost Benefit Analysis

This report presents a cost- benefit analysis for upgrading the existing decarbonizing of steel trays used in the Danish Pastry`s production facility. The current system has operational inefficiencies related to manual hard work, higher energy consumption, and inconsistent product quality.

The proposed upgrade introduces an automatic rinsing and drying system whereas there is all manual in the existing one. As well as this project reduces the number of laborers needed to operate the machine and risk associated with  the manual work.

Key assumptions

- Workday = 8 hours; year = 365 days (used for annual calculations)

- Electricity cost = CAD 0.141 / kWh

- Labour rate = CAD 20 / hour

- Maintenance = 5% of equipment investment per year

- Capital cost (investment) = CAD 29,728 (rounded from BOM totals)

- Energy use and labour inputs are as provided in the table

Current manual process leads to:

- Low throughput and variable cycle time

- High labour hours and labour risk (burns, fatigue)

- Inconsistent cleaning quality (rework / rejects)

- Frequent downtime (legacy PID controllers) and maintenance
Objectives of the upgrade:

- Double throughput (from 160 → 320 units/day)

- Reduce labour from two operators to one (or less monitoring)

- Improve product quality consistency and safety

- Reduce downtime and maintenance

Comparison of Key performance Indicators

| Existing System |  |  |  |
| --- | --- | --- | --- |
| KPI | Data | Metrics | Remarks |
| Throughput | 160 | Units/Day | 20 Pieces of trays per hour and equals 160 units per day/8 hours |
| Cycle Time | <60 | Minutes | Cycle time depends on how the worker works and is usually more than 60 min |
| Labour Hours | 16 | Hours | Two worker working for 8 hours a day, weekly=16*7=112 hours |
| Machine Utilization | 33.33% | % | Machine utilized for only 20 minutes every hour. |
| After Improvement/Automation/Upgradation |  |  |  |
| KPI | Data | Metrics |  |
| Throughput | 320 | Units/Day | 40 pieces of trays per hour and is equal to 320 units per day/8 hours. |
| Cycle Time | <30 | Minutes |  |
| Labour Hours | 8 | Hours/Day |  |
| Machine Utilization | 0.67% | % |  |

Cost of the Project (Bill of Materials)

| Category | Item Description | Specification / Details | Qty | UOM | Estimated Unit Cost (CAD) | Total Cost (CAD) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mechanical Components | Stainless Steel Decarbonizing Tank(SS316) | [8 ft x 4 ft x 4 ft]*2, SS304] | 72 | Sq.Ft | 44 |  | Decarbonizing Tank |
|  | Rinsing Tank | 8 ft x 4 ft x 4 ft, SS304 | 60 | Sq.Ft | 44 | 662 | Rinsing Tank |
|  | Guide rail and mechanical Assembly | 18 | 1 | Piece | 1,500 | 1,500 |  |
|  | Drying Chamber with hot air blowers | 8 ft x 1 ft x4 ft, SS304) | 72 | Sq.Ft | 44 | 792 | Drying Chamber |
|  | Conveyor System / Hoist Assembly | Motorized with frame supports | 1 |  | 6,000 | 6,000 | Load capacity 1 ton |
|  | Horizontal Hoist Drive System |  | 1 | Set | 502.10 | 502 |  |
|  | Float Valve | 1/4" | 1 | Set | 57.93 | 58 | Decarbonizing Tank |
|  | Structural Frame / Beams | Evergreen Styrene I Beam 3.2mm (0.125") | 1 | Set | 2,500 | 2,500 | For support & stability |
|  | Drain Valve | 1/2" | 3 | Piece | 37 | 2,500 | Decarbonizing Tank, Rinsing Tank and Drying Stage |
|  | Chemical Filtration Unit | filters and accessories | 1 |  | 58 | 58 | Automatic cycle filtration |
|  | S.S Pipe nipple 1/2"*4" | 1/2" | 22 | Piece | 6 | 124 | Decarbonizing Tank |
|  | S.S Pipe 1/4"*36" | 1/4" | 84 | Piece | 6 | 475 | Rinsing Tank |
|  | S.S Pipe 1/4" Tee | 1/4" | 84 | Piece | 9 | 756 | Rinsing Tank |
|  | S.S Pipe nipple 1/2" Tee | 1/4" | 21 | Piece | 8 | 168 | Rinsing Tank |
|  | Mechanical Componenets Sum(16095 CAD) |  |  |  |  |  |  |
| Electrical Components | Induction Motors(Hoist and Conveyor) | 1 HP / 3 Phase | 2 |  | 366.34 | 733 | For hoist & conveyor |
|  | Speed control pump | Grundfos 3-Speed 115 V 1/25 HP IB Nonsubmersible Circulation Pump | 1 | Piece | 216 | 216 | For Rinsing Tank |
|  | Induction Motors | 1 HP / 3 Phase | 1 |  | 650 | 650 | For Spraying Unit |
|  | Recirculation pump for filtration |  | 1 |  | 47.28 | 47 | Decarbonizing Tank |
|  | Electrical Box, IP67 Waterproof Hinged Cover Stainless Steel Latch Junction Box, | ABS Plastic Electrical Enclosure with Mounting Plate,Wall Brackets, Cable Glands,24.8"x20.8"x9.8",Grey | 1 |  | 230 | 230 | Electrical Panel |
|  | PLC Unit | Allen-Bradley CompactLogix | 1 |  | 2,200 | 2,200 | Electrical Panel |
|  | 2711P-T12W21D8S-B PanelView Plus 7 Standard 12 W Terminal | 7” Touchscreen | 1 |  | 1,000 | 1,000 | Electrical Panel |
|  | VFD 220V 2.2KW 3hp Variable Frequency Drive CNC VFD Motor Drive Inverter Converter for Spindle Motor Speed Control HUANYANG HY-Series | For motor speed control | 2 |  | 214.56 | 429 | Electrical Panel |
|  | 9 KW water Heater |  | 1 | Set | 99.99 |  | Decarbonizing Tank |
|  | MEAN WELL EDR-120-24 Single Output DIN Rail Power Supply 24V 5 Amp 120W |  | 1 |  | 41.35 | 41 | Electrical Panel |
|  | 16 Channel Relay Module, DC 5/12/24V 5mA Interface Board High/Low Level Trigger Small-Sized Two-Way Isolated Optocoupler Built-in Finite Current Resistor (DC24V) |  | 1 | Set | 15.53 | 16 | Electrical Panel |
|  | 50W Reed Proximity Switch Sensor | Position & safety sensors | 4 |  | 19.65 | 79 | Process feedback |
|  | RTD Temperature Sensor, RTD PT100 Temperature Sensor Probe 1/2" NPT Thread Thermocouple Terminal Head,Simple and Easy to Install(200mm/7.9in) |  | 1 | Piece | 22.72 |  | Decarboniing Tank |
|  | LAY37-11ZS Elevator Emergency Stop Mushroom Head Button (Red) |  | 2 | Piece | 8.995 | 18 | Electrical Panel |
|  | 50W Reed sensor, IP67 rating |  | 5 | Piece | 9.99 | 50 | Decarbonizing Tank |
|  | Analog Output Inductive Proximity Sensor | Cylindrical, 0-10V/4-20mA | 1 | Piece | 173.97 | 174 | Hoist |
|  | 5 KW Heat Strip with breaker |  | 1 | Piece | 172.34 | 172 | Dryer |
|  | Power supply Breaker MCCB | Miniature Circuit Breaker - S200 80-100A - 3P - 80 A - C - (AC) 6 kA | 1 | Piece | 92.53 | 93 | Electrical Panel |
|  | Control Wiring, Cables, Conduits | Power & signal wiring | 1 lot |  | 700 | 700 | Installation wiring |
|  | Electrical Componenets Sum(6633 CAD) |  |  |  |  |  |  |
| Automation & Software | Automation Studio License | Educational | 1 | set | 0 | 0 | Software |
|  | FactoryTalk License | Educational | 1 | set | 0 | 0 | Software |
| Labour & Installation | Fabrication & Installation | Mechanical assembly | 100 | hrs | 40 | 4,000 | Installaion and Commissioning |
|  | Electrical Installation | Wiring, panel setup | 20 | hrs | 40 | 800 | Installaion and Commissioning |
|  | Installation Tools on Rent |  | 1 | Set | 1,000 | 1,000 | Installaion and Commissioning |
| Miscellaneous & Contingency | Safety Equipment, Accessories | PPE and safety acccessories | 1 | Set | 1,200 | 1,200 | Installaion and Commissioning |
|  | (Software, Labor/Installation,Misc=7000 CAD) |  |  |  |  |  |  |
| TOTAL |  |  |  |  |  | 29,942 | ≈ within proposal budget |

Operating Cost

| Item | Existing system | Cost of Existing System(CAD) | Upgraded system | Cost of Upgraded System(CAD) |
| --- | --- | --- | --- | --- |
| Energy Consumptiopn | (=9kw*8hr)=72 units/day, $ 0.141 per Kwh | 3705.48 | (=11.25 Kw*8hr)=90 units/day | 4631.85 |
| labor Cost | (=8hrs*20Phr*2W)=320 CAD/Day | 116800 | (=8hrs*20Phr*1W)=160 CAD/Day | 58400 |
| Maintenance Cost | 5% of Investment Annually,Cost of Machine Calculated=3300 | 165 | 5% of Investment Annually, Investment 29728 | 1486.4 |
| Total Operating Cost |  | 120670.48 |  | 64518.25 |

Annual Savings after upgrading

=Existing Operating cost - Operating Cost after Upgradation

=120670.48 - 64518.25

=56152.23 CAD (Canadian Dollar)

Benefits Analysis

- Energy Efficiency

Decarbonizing tank productivity will increase as it was operated manually and was depending on manpower. After automatic control and upgradation, productivity will be doubled from 20 to 40 minutes per hour. This also reduces the damage of S.S trays as they will be inside the tank at higher temperatures only for specified periods. Previously, trays were inside the decarbonizing tank for longer period, and it was causing bent on the trays.

- Reduced Labor Requirement

Less manual monitoring due to automation. Just 5-10 minutes an hour is required in actual.

Annual Savings=116800-58400=58400

- Improved Quality Consistency

As manual rinsing quality was differing from one cleaner to another cleaner and was not perfect always, the rinsing machine cleans each trays area from less than 2.5 feet in distance with 3 bar pressure.

- Reduced Downtime

As old type of PID controller fails frequently, using of latest PLC technology for monitoring of temperature and heater operation as well, there will be less downtime as compared to existing system.

- Safety Improvement

Lower risk of burns and operator fatigue.

- Financial Analysis

Payback Period

Payback period= Capital (Investment) Costs/Annual Savings

=29728 /56152.23

=0.53 years

The payback period is approximately 6 to 7 months.

Return on Investment (ROI)

= (56152.23-29728)/29728

=0.88

=88%

ROI increases in future years, as it is 88% in the year of investment.

Conclusion

The proposed automation of the decarbonizing, rinsing and drying line yields large operational and financial benefits. With a capital outlay of CAD 29,728 and projected annual savings of CAD 56,152, the project pays back in ≈6–7 months and produces an excellent ROI. Benefits include higher throughput, improved quality, safer working conditions and much lower labour exposure. Based on this analysis, the upgrade is strongly recommended.